Red Rhino 4000 Concrete Crusher – Hirer Instructions

Purpose

This guide explains how to use the Red Rhino 4000 safely and correctly during your hire with Kougar Tool Hire. It highlights prohibited materials, essential operating rules, and responsibilities while the machine is in your care.

What You MUST NOT Put in the Crusher

- No rebar, mesh, or reinforced steel — remove all metal before crushing.

- No timber, plastics, rubber, or general waste — concrete and brick only.

- No glass, ceramics, or tiles — these can shatter and damage internal components.

- No oversized material — maximum piece size: 400mm x 170mm.

- No wet slurry or soil — the crusher is not designed for mud or loose earth.

- No hazardous materials — including asbestos, chemicals, or contaminated waste.

Essential Operating Rules

- Machine must be operated by a competent adult (18+).

- Keep hands, feet, and clothing away from moving parts at all times.

- Always use the emergency stop if anything becomes jammed or unsafe.

- Feed material slowly and evenly — do not overload the hopper.

- Ensure the crusher is on level, stable ground before starting.

- Check the conveyor is clear before switching on.

- Wear mandatory PPE: hard hat, gloves, steel-toe boots, eye protection, and ear protection.

- Maintain a clear exclusion zone around the machine (minimum 3 metres).

- Never leave the machine running unattended.

Do’s for Safe Use

- Do inspect material before feeding to ensure it is clean and metal-free.

- Do keep the work area tidy to avoid slips and trips.

- Do monitor the output size and adjust jaws only when the machine is off.

- Do report any faults immediately to Kougar Tool Hire.

- Do shut down properly: stop feed, idle to clear, power off, isolate.

Don’ts for Safe Use

- Don’t force material into the jaws.

- Don’t climb on the machine or stand on the conveyor.

- Don’t attempt repairs or adjustments while the machine is running.

- Don’t use the crusher in poor visibility or without adequate lighting.

- Don’t allow untrained persons to operate or assist.

Hirer Responsibilities

- You are responsible for the machine from delivery to collection.

- Any damage caused by misuse or prohibited materials will be chargeable.

- Daily checks (fluids, belts, safety guards) must be carried out.

- Fuel used must match specification provided at handover.

- Machine must be cleaned before collection.

Emergency & Faults

- Use the emergency stop button immediately if unsafe conditions arise.

- Stop the machine, isolate power, and do not attempt to clear blockages by hand.

- Contact Kougar Tool Hire for support on 0208 875 1177

Kougar Tool Hire – Safe, Reliable, Professional